AIM COMPOSITES MATERIALS DATABASE: AN AI-ASSISTED REPOSITORY

FOR COMPOSITE THERMOPLASTIC PROPERTY DATA

by

Mathias Heider

A project report submitted to the Faculty of the University of Delaware

in partial fulfillment of the requirements for

ME8930 — AI for Composites and Manufacturing

Spring 2026

© 2026 Mathias Heider

All Rights Reserved

ABSTRACT

We present the AIM Composites Materials Database. This is an open-source repository for experimental property data on polymers, fibers, and fiber-reinforced composites. Our database holds over 1,500 material records that go over 445 property types. We built the system using PostgreSQL, a Streamlit frontend which is deployed on Hugging Face Spaces, and an extraction pipeline that converts PDF datasheets and research papers into structured database rows using LLMs like Google Gemini. In this report, we will describe our database design, system architecture, and user workflows. We also introduce a prototype that runs our extraction and validation pipeline on a folder of PDFs without human involvement using autonomous AI. This work is a joint effort between Clemson University and the University of Delaware.

Chapter 1

INTRODUCTION

Machine learning for composites engineering depends on good training data. This includes data for property prediction, inverse design, and process optimization. For thermoplastic composites, that data does not exist in a usable form. Property data is scattered across manufacturer datasheets and journal papers in PDF format and is not easy to access or query. Existing databases like MatWeb [19] and Ansys Granta MI [21] cover general polymer properties, but they have limited coverage of continuous fiber-reinforced thermoplastic laminates and almost no data on how processing conditions affect material properties. For more than a decade, NASA, FAA, industry, and academia have worked to build a central composite database. The result is the Composite Materials Handbook-17 [20], but it is not freely available in a queryable format and focuses mostly on thermoset systems certified for aerospace [22]. No widely accepted open repository for thermoplastic composite property data currently exists.

The AIM Composites Materials Database is our solution to this problem. Our goal is to give researchers a queryable, open repository of polymer, fiber, and composite property data. We also want to use that data to generate training sets for machine learning models in the AIM initiative, including property prediction, process quality modeling, and structural performance simulation.

Building a database like this is not just a data management problem. It is also a problem in AI for manufacturing. Manually extracting thousands of property values from PDFs is too slow to scale. The data also needs test conditions, units, and source information to be useful for machine learning. To solve this, we use a large language model pipeline to convert PDFs into structured database rows automatically. We will describe our database design in Chapter 3, our system in Chapter 4, and our user workflows in Chapter 5. We will then go over our current data in Chapter 6, our autonomous prototype in Chapter 7, and our limitations in Chapter 8.

Chapter 2

RELATED WORK

2.1  Automated Property Extraction

The problem of getting structured data out of scientific PDFs has been studied for a while. ChemDataExtractor [1], developed by Swain and Cole, was one of the first tools built for this. It uses a pipeline of text processing steps to pull chemical entities and property values out of papers and gets F-scores of 93.4% for chemical identifiers and 91.5% for property values. Later versions extended the approach to materials science more broadly [2], and researchers have used it to build large databases automatically. One example is a database with over 720,000 stress-strain records pulled from the scientific literature [2]. A similar pipeline from the Ramprasad group at Georgia Tech used transformer-based models to extract polymer properties from a large collection of papers [3].

Large language models have changed how this is done. Instead of writing custom rules for each property type, we can now just prompt a model and it figures out different table layouts, unit conventions, and caption styles on its own. PropertyExtractor [4] uses GPT-4 and Gemini-Pro with engineered prompts to extract and verify material properties automatically. A benchmarking study by Liang et al. [5] found that GPT-4 models get an F1 of about 0.91 for thermoelectric property extraction, while Gemini models perform less consistently across different property types. Forcing the model to output a fixed JSON structure instead of free text was shown to significantly reduce errors [5, 6]. More recent work has gone further by building full agent pipelines. Banik et al. [7] process around 10,000 full-text papers using multiple agents that check each other for consistency. Cheng et al. [8] built a system that handles text, tables, and figures together using a layered prompting strategy.

2.2  Machine Learning for Composite Properties

Our database exists to support machine learning models that predict composite properties. This is a growing area of research. Liu et al. [15] used gradient boosted decision trees to predict the tensile strength of 3D-printed carbon fiber PEEK composites from printing parameters and got an R-squared above 0.8. Fang et al. [16] used neural networks for multilayer thermoplastic composite design. More broadly, XGBoost has become a common baseline algorithm for tabular composite property data [13, 14]. A problem that shows up in all of these studies is that training datasets are small. Most models rely on a few hundred to a few thousand data points collected by hand. The lack of a large open repository is consistently cited as the main bottleneck [13, 14]. This is exactly what our database is trying to fix.

2.3  Database Design for Sparse Property Data

To store our data, we use a design where each property observation is stored as its own row. This approach is well established in medical informatics, where similar challenges come up with heterogeneous and sparse data [17, 18]. Nadkarni et al. [17] show that this kind of design works well when the set of properties you want to track is open and growing and when each observation has its own metadata, both of which are true for composite material data. Dinu and Nadkarni [18] look at performance at large scale and identify indexing strategies to keep queries fast, which we apply to our own database.

Chapter 3

DATABASE DESIGN

3.1  How We Organize Materials

We organize our database around three types of materials. The first is Polymers, which covers neat thermoplastic and thermoset matrix materials like PEEK, PEI, PA66, ABS, PTFE, and PEKK. The second is Fibers, which includes reinforcement materials like carbon fiber, E-glass, S-glass, aramid, basalt, and natural fibers. The third is Composites, which covers fiber-reinforced laminates and molding compounds. Each composite entry describes the matrix, fiber type, fiber volume fraction, layup architecture, and processing route.

Within each material type, we group properties into sections. These are Mechanical, Thermal, Electrical, Physical, Optical, Rheological, Processing, and Descriptive. For composites, we also have Composition and Architecture sections. Each section has a list of common property names. Users can also enter a free-text property name if theirs does not fit the list. This keeps things consistent and makes cross-material queries work well. Right now we track 445 different property types across all three material types.

3.2  How We Store the Data

We store each property observation as its own row in the database. Table 3.1 shows the columns we use. We chose this design over a wide table with one row per material and one column per property for three reasons. First, composite data is sparse. Any given datasheet only reports a small fraction of all possible properties, so a wide table would be mostly empty. Second, our list of properties is always growing. Adding a new property to a wide table would require changing the database structure. Third, each property observation has its own test conditions and we want to keep that information together with the value. The downside is that a query like "give me the tensile modulus for every PEEK grade" has to filter by property name instead of just picking a column. This is fine at our current size and gets faster with the right indexes [17, 18].

| Column | Description |
| --- | --- |
| material_name | Generic material name (e.g., "Polyetheretherketone") |
| material_abbreviation | Short name used as a key (e.g., "PEEK") |
| section | Property group (Mechanical, Thermal, Electrical, etc.) |
| property_name | Name of the property |
| value | Measured value or range, stored as text |
| unit | SI unit (e.g., "GPa", "degrees C", "g/cm3") |
| english | Imperial unit value if reported in the source |
| test_condition | Test temperature, strain rate, standard, sample geometry |
| comments | Notes, footnotes, and standard references |

Table 3.1: The columns we use in all three material tables.

3.3  Source Information

Right now our database does not have a column for where the data came from. There is no DOI, PDF name, or page number stored in a way that can be searched. Sometimes the extraction pipeline picks up source information and puts it in the comments column, but that does not happen consistently. This is a real problem because without structured source information, we cannot trace a property value back to where it came from. This makes the data harder to verify and reuse. Adding a sources table with a link to every property row is our top priority going forward, and we discuss this more in Chapter 8.

Chapter 4

SYSTEM ARCHITECTURE

4.1  Overview

Our system is made up of three main parts. For storage, we use a PostgreSQL database hosted externally. We pass the connection details to the application through environment variables so credentials stay out of the code. The application is a Streamlit web app with four pages: Home, Categorized Search, Upload Data, and Contact Team. We deploy it on Hugging Face Spaces in a Docker container based on python:3.11-slim running on port 7860. The application and source code are publicly available at huggingface.co/spaces/aim4composites/MaterialsDatabase and github.com/AIM-for-Composites/InDesDatabaseTool.

4.2  How We Extract Data from PDFs

Our PDF pipeline is built around Gemini 2.5 Flash. To extract properties from a PDF, we encode the file and send it to the Gemini API. We set temperature to zero so we get consistent results every time. We also give the model a fixed JSON structure that it has to fill out. The structure includes fields for the material name, abbreviation, trade grade, and manufacturer at the top level. For each property, we ask for the section it belongs to, the property name, its value, its unit, an imperial unit version if available, the test condition, and any comments. We prompt the model as an expert materials scientist and ask it to pull out all properties across all categories into one list. The pipeline returns a table of results that matches our database structure, or nothing if the API call fails.

We also extract figures from each PDF using PyMuPDF and OpenCV. We convert each page to an image at 300 DPI, use thresholding and edge detection to find regions that look like plots, and match each one to a nearby caption. For property name matching, we use the all-MiniLM-L6-v2 sentence transformer model [9] to find how similar two property names are using vector embeddings. We then take the top matches and verify them with a Gemini yes or no prompt. This approach is consistent with recent high-accuracy extraction systems [5, 6].

Chapter 5

USER WORKFLOWS

Our application has three main ways users can interact with it. The first is browsing. Users go to the Categorized Search page, pick a material type, and can filter by matrix or fiber type. They search by name or abbreviation and get results in a table of 50 rows at a time. Clicking a row shows the full property data and any figures we have stored for it. The second workflow is manual entry. Users fill out a form to add one property row at a time. This is useful for targeted additions or data that the PDF pipeline cannot handle well. The third workflow is PDF upload. Users drag in a datasheet or paper and our pipeline runs on it automatically. The extracted rows are shown for review, and the user picks the material type and approves before anything goes into the database. Our figure extraction pipeline also runs at the same time so users can link figures to specific properties. Right now a person still needs to review and approve the extracted data. Our autonomous prototype in Chapter 7 is aimed at removing that step.

Chapter 6

CURRENT DATA AND QUALITY

6.1  What We Have

Our database currently holds over 1,500 material records and tracks 445 different property types. The data covers all three material types and was added through a combination of manual entry, PDF uploads, and batch ingestion from manufacturer datasheets.

6.2  Data Quality Issues

Going through our data, we found five issues that come up regularly. The first is unit inconsistency. Some rows use an empty unit for dimensionless quantities like Shore D hardness, while others leave the unit field out entirely. Some value strings also have the unit embedded directly inside them, like "1.50 J/cm NB," which breaks numeric parsing. The second issue is mixed unit systems. The value column is supposed to hold SI values with imperial equivalents in the english column, but some rows mix both in one string. The third issue is multiple rows for the same property at different test conditions. This is actually correct behavior since the same property can have different values at different temperatures, but it means we cannot use a simple uniqueness rule on material plus property name. The fourth issue is that some entries store things like average values and grade counts as free text in the comments column instead of in dedicated fields. The fifth and most important issue is that we have no structured source information. These issues all led us to build a validation layer into the autonomous prototype we describe in Chapter 7.

Chapter 7

TOWARD AUTONOMOUS POPULATION

Right now our ingestion pipeline still needs a person in the loop. A human supplies each PDF, picks the material type, and reviews the extracted rows before anything goes into the database. Our goal is to build a system that can do all of this automatically. It would find papers to ingest, extract and validate the data, and either insert it directly or flag edge cases for a person to review.

7.1  The System We Want to Build

We break the target system into four steps. The first is finding papers. Given a search goal like "PEEK carbon fiber unidirectional tensile data," an agent would query Semantic Scholar, OpenAlex, and arXiv, rank the results by relevance, and download the top ones. The second step is extraction, which uses our existing Gemini pipeline. It already works without human input for most PDFs. The third step is validation. Each extracted row gets checked for unit issues, whether the value is in a reasonable range for that property type, and whether it already exists in the database based on the material name, property name, and test condition. Rows that fail go to a review queue with a reason attached. The fourth step is adding the data to the database. Every row that passes validation goes in, and each one will also store a link to a source record that includes the PDF hash and DOI if available.

7.2  Our Prototype

We implemented a batch ingestion script called batch_ingest.py that covers steps two through four on a fixed folder of PDFs with no human involvement. The script goes through each PDF in order and calls our Gemini extractor on it. It then assigns a material type by checking the extracted material name for keywords. If words like "fiber," "carbon," or "glass" show up, it routes the material to Composite or Fiber. Otherwise it goes to Polymer. Each row then goes through our validation pipeline, which checks units, parses the value numerically, checks whether the value is in a reasonable range, and looks for duplicates. Rows that pass go into a local SQLite copy of our database so we can test without touching the live system. Rows that fail go to a CSV file with a reason code. We left out paper discovery and figure extraction from this prototype. The goal was to understand how the extraction and validation loop performs end to end before we build the rest.

7.3  Results

We tested the prototype on a set of polymer datasheets. Each PDF took between 10 and 25 seconds to process, with most of the time spent waiting on the Gemini API. Our validation flags fell into three groups. About 15% of rows were flagged for missing units, but most of these were false positives for properties that are correctly unitless, like Shore D hardness. Around 5 to 10% of rows had units embedded inside the value string, which is a real problem introduced by Gemini. A small number of rows had values outside the expected range, usually because the model misread scientific notation or a unit prefix. Deduplication worked correctly. Re-running the same PDF inserted zero new rows. These results show that the pipeline can run on its own at a manageable cost and that the main failure modes are fixable. Our next step is to build a hand-labeled set of ground truth data so we can measure how accurate our extraction really is.

7.4  What We Plan to Do Next

There are five things we want to do to move the prototype toward a production system. First, we need to add a sources table and store structured source information for every row we insert. Second, we want to build a hand-labeled ground truth set of 50 to 100 rows and measure extraction accuracy against it. Third, we want to replace the keyword-based material type assignment with a small trained classifier. Fourth, we plan to add paper discovery using OpenAlex, Semantic Scholar, and arXiv so the system can find its own papers to ingest. Fifth, we want to move our validation rules into a config file so domain experts can adjust the expected value ranges without touching any code.

Chapter 8

LIMITATIONS AND FUTURE WORK

Our system has several known limitations. The most important is that we do not have structured source information. Until we add a sources table, property values cannot be traced back to where they came from. This limits how useful the database is for reproducible research and means we do not yet meet open data standards [10]. This is our top priority going forward.

Our value column stores data as text. This lets us handle ranges, qualifiers like "NB" for non-break, and multi-condition entries, but it makes numeric queries and statistical summaries hard without extra parsing. We plan to add a separate numeric column alongside the raw text value to fix this.

We also do not have versioning or access control. Updating a record overwrites it with no history saved. Any user can add data to the database without a review step. This will become a real problem as more people use the system. Gemini also gets noticeably worse on scanned PDFs, complex multi-axis plots, and unusual table layouts. These are the hardest cases in our dataset and will need better preprocessing or prompting to handle well.

Finally, we use the material abbreviation as the identity key for each entry. This is brittle at scale because two entries for the same material from different sources might use different abbreviations, creating duplicates that a simple string match will not catch. As the database grows, we will need a smarter way to identify materials. Going forward, we want to close the autonomous ingestion loop, add the provenance layer, add numeric value parsing, and build a reviewer approval step. Most importantly, we want to use this database to train and test the machine learning models it was designed to support.

REFERENCES

[1]  M. C. Swain and J. M. Cole, "ChemDataExtractor: A toolkit for automated extraction of chemical information from the scientific literature," J. Chem. Inf. Model., vol. 56, no. 10, pp. 1894-1904, 2016. doi: 10.1021/acs.jcim.6b00207

[2]  J. Mavracic, C. J. Court, T. Isazawa, S. R. Elliott, and J. M. Cole, "ChemDataExtractor 2.0: Autopopulated ontologies for materials science," J. Chem. Inf. Model., vol. 61, no. 9, pp. 4280-4289, 2021. doi: 10.1021/acs.jcim.1c00446

[3]  P. Shetty, A. C. Rajan, C. Kuenneth, S. Gupta, L. P. Panchumarti, L. Holm, C. Zhang, and R. Ramprasad, "A general-purpose material property data extraction pipeline from large polymer corpora using NLP," npj Comput. Mater., vol. 9, p. 52, 2023. doi: 10.1038/s41524-023-01003-w

[4]  A. Gupta, R. Hoover, and J. Brgoch, "Dynamic in-context learning with conversational models for data extraction and materials property prediction," arXiv:2405.10448, 2024.

[5]  Q. Liang, A. E. Gongora, Z. Ren, A. Abdel-Latif, K. G. Reyes, P. R. Brown, S. Srinivasan, and T. Buonassisi, "From text to insight: Large language models for materials science data extraction," arXiv:2407.16867, 2024.

[6]  J. Dagdelen, A. Dunn, S. Lee, N. Walker, A. S. Rosen, G. Ceder, K. A. Persson, and A. Jain, "Structured information extraction from scientific text with large language models," Nat. Commun., vol. 15, p. 1418, 2024. doi: 10.1038/s41467-024-45563-x

[7]  S. Banik, W. Chen, K. Choudhary, and F. Tavazza, "Automated extraction of material properties using LLM-based AI agents," arXiv:2510.01235, 2025.

[8]  Y. Cheng, H. Zhang, and Y. Sun, "From papers to property tables: A priority-based LLM workflow for materials data extraction," arXiv:2604.07584, 2025.

[9]  N. Reimers and I. Gurevych, "Sentence-BERT: Sentence embeddings using Siamese BERT-networks," in Proc. EMNLP, 2019. doi: 10.18653/v1/D19-1410

[10] M. Scheffler et al., "Community action on FAIR data will fuel a revolution in materials research," MRS Bull., vol. 48, pp. 1024-1040, 2024. doi: 10.1557/s43577-023-00498-4

[11] National Science and Technology Council, "Materials Genome Initiative Strategic Plan," 2021. Available: https://www.nist.gov/system/files/documents/2021/11/17/MGI-2021-Strategic-Plan.pdf

[12] C. H. Ward, "Materials Genome Initiative: Materials data," NISTIR 8038, NIST, 2014. doi: 10.6028/NIST.IR.8038

[13] A. Dima et al., "Machine learning-driven prediction of composite materials properties based on experimental testing data," Polymers, vol. 17, no. 5, p. 694, 2025. doi: 10.3390/polym17050694

[14] S. Han, H. Zhang, and J. Chen, "Toward accurate machine learning-driven prediction of polymeric composites properties," Mater. Genome Eng. Adv., 2025. doi: 10.1002/mgea.70027

[15] P. Liu, B. Chang, C. Xu, W. Zhang, T. Yang, and T. Wu, "Optimizing 3D printed continuous CF/PEEK composites: A machine learning approach to strength prediction," J. Reinf. Plast. Compos., 2025. doi: 10.1177/07316844251356346

[16] R. Fang, L. Wu, and Z. Sun, "Machine learning-assisted design of multilayer thermoplastic composites," Macromol. Mater. Eng., 2025. doi: 10.1002/mame.202500093

[17] P. M. Nadkarni, L. Marenco, R. Chen, E. Skoufos, G. Shepherd, and P. Miller, "Organization of heterogeneous scientific data using the EAV/CR representation," J. Am. Med. Inform. Assoc., vol. 6, no. 6, pp. 478-493, 2009.

[18] V. Dinu and P. M. Nadkarni, "Exploring performance issues for a clinical database organized using an entity-attribute-value representation," J. Am. Med. Inform. Assoc., vol. 14, no. 1, pp. 60-65, 2007.

[19] MatWeb LLC, "MatWeb material property data," 2024. Available: https://www.matweb.com/

[20] CMH-17 Coordination Group, "Composite Materials Handbook-17," SAE International, 2024. Available: https://www.cmh17.org/

[21] Ansys Inc., "Ansys Granta materials data," 2024. Available: https://www.ansys.com/products/materials

[22] CompositesWorld, "Shared composite material property databases," 2024. Available: https://www.compositesworld.com/columns/shared-composite-material-property-databases